The program CLKCOMM.EXE is a DOS program that handles communications with the

InTagral clock through a standard PC serial port under the control of a calling

program.

The interface with the caller is handled through text files and DOS command

line paramet ers. File commands consist of a text line with the first character

being the command identification code optionally followed by a delimiter

haracter and command parameters.

Input to the program

CLKMNG.PAR

This file defines the environment. It is the same file used by CLKMNG.EXE with

a few added  options. The file name may be overriden using a DOS command line

parameter (see below).

The relevant commands are:

B 9600          Sets baud rate (2400,4800,9600), default 9600

P 2             Sets Comm port number (1..4), default 1

N <filename>    Name of the file for writing transaction data

downloaded from InTagral, default SEND1.TXT.

F <formstr>     Sets the format of transaction data written to the

Output file. See below for details on formstr

{Y+}-{M}-{D}-{W}-{A1|0}-{h}:{m}{TI|O|X|Y}{L}

W <filename>    Name of a file that holds commands for clkcomm to

execute, default CLKCMD

X <filename>    Name of output file for results of clkcomm other than

transaction data. If filename is empty clkcomm will

write to the screen, default CLKSTT.

R               Modem initialize string.

J               Dial number string

!               By default clkcomm displays progress on the screen for

commands that take time such as downloading data or

uploading program code. This command will silence this

display.

CLKCMD

This file holds commands for clkcomm.exe to perform. The file name may be

changed by a command in CLKMNG.PAR or by a DOS command line parameter.

Single commands can be executed using a DOS command line parameter instead of

this file.

The commands available:

R               Download new transaction data from InTagral. The output

file name (default SEND1.TXT) and data format are defined

in CLKMNG.PAR. If the file exists it is overwritten.

A <DDMMYY>      Download all transaction data from InTagral. InTagral

retains data that has been previously downloaded in

circular memory but the regular download command (R)

does not retransmit. If the date parameter is omitted all

stored data is sent otherwise only data with date newer

or equal. The data file used is the same as in R.

G               Get InTagral status information. Data is written to the

secondary output file (default CLKSTT) see example below.

C #             Set Intagral time to PC clock time.

C DDMMYYhhmmss  Set Intagral time (hh in 24 hour format).

N XXXX          Sets InTagral clock ID

M n1,n2,n3,n4,n5   Sets Employee tag parameters.

n1 = minimum characters,

n2 = maximum characters,

n3 = characters to skip,

n4 = No. of characters in ID field.

n5 = characters to skip for display field,

(*=the 3 right most char. from the ID field

as defined by n3 and n4)

IMPORTANT!!!  Any change to parameter n4 will clear data,

destroying all stored transactions.

I <TI1,TO1,TI2,TO2,TI3,TO3,TI4,TO4>

Sets In/Out time table, TIn = hhmm in 24 hour format.

V <i:d-TIn ... i:d-TIn>

Sets In/Out times for special days i=1,2 first or second

special day, d=1..7 sunday..saturday. TIn=time table as in

I command

D ddmm,ddmm     Sets on and off date for daylight savings time

Put ---- instead of ddmm for no date set.

O <XYTUDC>      enable special operations X, Y,

fixed override (T),

Auto Exit/Entry (U),

enable Duplicate (D),

Consecutive operation (C).

if the letter does not appear the operation is disabled.

P               Get version numbers of code file in PC. Used for

checking if InTagral firmware needs to be updated.

See example below.

U               Update InTagral firmware. Code is divided into 2 parts,

Boot and Application, both parts are held in the file CLOCK.HEX.

Each part has a seven character version ID. The version numbers

in Intagral are reported using command 'G', those in the file

CLOCK.HEX are reported using command 'P'.

The command 'U' will upload only in case version ID does not

match.

H STORAGE

DOS Command Line parameters

Commands can be set by adding parameters to the DOS command line that invokes

clkcomm.exe. The values in these parameters override those set in the files.

The format of commands is similiar to file commands but there is no delimiter

following the command ID and several parameters can be on the same line with a

space in between. Valid parameters:

B9600

P2

N<filename>

F<formstr>

W<filename>

X<filename>

!

+<command>

@<filename>

All parameters except the last two are identical to those in CLKMNG.PAR.

the + parameter specifies a command directly instead of using a command file.

The format of the command given in the + parameter is identical to that for

CLKCMD above except that the delimiter between the command ID and command

parameters cannot be a space. The @ parameter changes the name of of the

parameter file from CLKMNG.PAR to filename.

CLOCK.HEX

This is object code for InTagral. Used for firmware updating.

Output of the program

SEND1.TXT

Transaction data downloaded from InTagral. A transaction record sent by InTagral

has the following fields:

Operation type - Entry, Exit, X or Y

Date: Day (DD), Month (MM), Year (YY)

Time: Hour (hh), Minute (mm), Second (ss)

Tag ID - Number of characters defined in parameter M.

These fields can be combined in any order, together with constants and with

Intagral ID or serial No. to produce the output record format. The format is

defined by a text string in the 'F' command.

This string is a series of constants and formatting instructions inside curly

brackets {}. Each instruction is identified by its first letter (following the

opening bracket), optionally followed by parameters that are divided by a comma

or '|' character.

Available instructions are listed below:

{TI|O|X|Y|U|M|?} - The operation type field. The characters I,O,X,Y,U,M,?

are used to represent the values of Entry, Exit, X, Y, and U undefined,

M Master card.

{D} - The 2 digit day of month field.

{M} - The 2 digit month field.

{Y} - The 2 digit year field (95).

{Y+} - Year field in 4 digits (1995).

{h} - The 2 digit hour field in 24 hour format.

{m} - The 2 digit minute field.

{s} - The 2 digit seconds field.

{An|l} - The tag ID field. The parameters n and l make it possible

to split the ID into parts. n is the starting point within the ID field,

(a value of 1 means start from the begining, 2 means skip one character).

l is the number of characters to output, a value of 0 is used to denote all

available characters up to the end of the field (if l is bigger than the

actual field leading zeroes are added).

{Zn} - The Intagral ID. n is optional and is used to limit the length

of the output field to less then the maximum of 4 (the last n characters

are output).

{Sn} - Intagral serial number, maximum 6 digits. n as in instruction Z.

{L} - Outputs a carriage return (hexadecimal 0D) followed by a line-feed

(hexadecimal 0A). Will normally be placed at the end of formstr.

{Cnn} - Outputs a single character with code nn in decimal. Further codes

may be added with comma in between. For example {C13,10} is equivalent

to {L}.

The default format, used if formstr is empty, is equivalent to the

following: {TI|O|X|Y|?}{D}{M}{Y}{h}{m}{s}-{A1|0}{L}

CLKSTT

The secondary output file is used to report program start and stop times

(from PC clock), error messages, and data for 'G' and 'P' commands.

Examples

type:

clkcomm +P <Enter>

the file CLKSTT should look similiar to this:

1 T-080395192054 V-08/03/95

P APP-F260195 BOOT-B260195

9 T-080395192054

The first line gives starting time and clkcomm version. The second line gives

version data from the file CLOCK.HEX. Intagral firmware is divided into two

sections for boot and application. The third line gives ending time.

type:

clkcomm +G Xclkstt1 <Enter>

File clkstt1 should be created and look like this:

1 T-080395192637 V-08/03/95

Z ID-0108 VER-F260195 SER-010826 BOOT-B260195

T 080395192648

B 0

F 0,0,11699

M 3,3,0,3

I 0815,1530,1700,1945

V 1:6-0700,1155

D ----,----

O X

9 T-080395192638

Z - ID version and serial numbers of InTagral

T - InTagral time

B - Battery state 0-OK, 1-LOW, 4-Lithium Low, 5-Both low.

F - n1,n2,n3 - number of transactions: to send, stored in memory, free space

M - n1,n2,n3,n4 - Tag parameters: max,min,skip,length.

I - In/Out time table.

V - Special days In/Out time table.

D - Daylight saving time On,Off dates

O - Special operations enabled.

Errors are indicated by a line in the following format:

E nn L-i error text

nn is the error code and i is the command line number

possible errors:

2 Serial Port i not defined

3 Missing command file

4 Bad Command

5 Comm Error

6 Unable to create data file

7 Error writing data to disk

8 Code file not found

9 First download data (for firmware update)

10 Disk read error

11 User Abort

12 Storage failed

The error code will also be set in the DOS errorlevel when exiting clkcomm.

Error 1 signals a  problem with the status file (CLKSTT).

type:

clkcomm +R <Enter>

data should be downloaded to SEND1.TXT

If InTagral is not connected then clkstt will look like this:

1 T-080395194130 V-08/03/95

E 5 L-1 Comm Error

9 T-080395194130

Additional commands available to clkcomm (either in the file CLKCMD or

specified in the command line parameter starting with "+").

r n,min<,max>   Check field in range

n-skip value for field position

min - string of up to 6 chars

max - string, same number of chars as max (default=min)

l iiioooxxxyyybbbrrrdddfffnnn

Messages displayed on LCD

3 chars per message for IN,OUT,OP1,OP2,BAD,REJ,DUP,FUL,NOT

Use '@' for space.

examples:

r 1,00,02

InTagral will accept cards that have chars 2,3 equal to 00, 01, or 02.

r

cancel range selection

l IN@OUTOP1OP2BADREJDUPFULNOT

When Intagrals are connected to the PC via a multidrop line then each session

requires a polling address consisting of a 6 character string equal to the

Serial number of the polled clock. The poll address  is given on the command

line preceded by the character S.

u S             Upload Confirm List

must be used with a parameter: & filename,Length,filepos,skip

Filename = Name of the text file containing the list

Length = Length of confirm field

Filepos = Number of chars to skip for relevant field in file

Skip = Skip to position in Tag

u D             Delete Confirm List

o nn            Open door time:

0 = No open

1-200 = Time to open - 10nn miliseconds

201-250 = Time to open - nn-200 seconds

examples:

clkcomm +G S000001

Get status of clock with serial number 000001

clkcomm +G S******

Get status of single clock connected over multidrop line.

clkcomm &EMPLIST.TXT,3,6,0 +u-S

Send list from file EMPLIST.TXT using columns 7-9